Tentacle Mechanism Cinching Instructions:

joining the tentacle to the controller

Prepare your workspace.
Let’s start with the upper stage.
			
Tape down the tentacle (not shown). Take two of the control cables that move the tentacle up and down along the same axis. Tape down their spring guides. Loop about 30[cm] of cable through the first hole.

Route the vented screw.

Slide the cable through the vented screw. Then slide the cable through the heat-set insert.
  
Do the same for the other side.

Reorient the pulley for Cinching

Tape down the controller such that it is in front of you. Prepare 2 clipped washers and 1 shoulder screw.

Pinch the right cable. Then drop it into the slot. Smush a clipped washer into the slot and onto the cable such that the hole is still exposed.

Next, Pinch the other cable and do the same from the other side.

Ensure that the hole is free of cable. Use a hex key to clear the hole so that the space is clear.

Insert the shoulder Screw. Do not let it catch the cable as you push it in. Screw it down about 2 turns, but don’t tighten it. Now, take up the cable slack by tugging both loose ends of the cable. Then tighten down the screw. (Your cables don’t have to be super tight for now as we can re-adjust later by simply loosening the screw.)

Ta-daa! Now set it aside, and do the same for the upper stage as shown below.